CONSTRAINT-BASED PROBLEM SOLVIN

CO5 ASSESSMENT 1

ITA0502 – COMPUTER VISION

Submitted by

M Yasodha Krishna (192472100)

# Real-Time Face Recognition for Campus Security

## Figure: OpenCV-based system diagram

## 1. Problem Statement

A university requires a real-time security system that captures faces from CCTV cameras, detects faces under different lighting conditions, extracts facial features, and identifies registered students and staff accurately while running on a low-cost computer.

## 2. Objective

To design an OpenCV-based face recognition pipeline that provides accurate and fast identification with minimum false recognition.

## 3. Constraints

- Real-time video processing.

- Different lighting conditions.

- Multiple faces in a frame.

- Limited computational resources.

- Minimum false recognition.

- Temporary face angle and movement variations.

## 4. Input

- Live CCTV/video stream.

- Registered face images of students and staff.

- Person ID/name database.

## 5. Processing / Algorithm

- Capture frames from the CCTV camera using OpenCV.

- Resize the frame to reduce computational cost.

- Convert the frame to grayscale.

- Improve illumination using histogram equalization or CLAHE.

- Detect faces using Haar Cascade or OpenCV DNN face detector.

- Crop the detected face region.

- Normalize the face size and alignment.

- Extract facial features using LBPH or a face embedding model.

- Compare the extracted features with registered face features.

- Apply a confidence/distance threshold to reduce false recognition.

- Display the recognized person's name and confidence on the video.

- Store unknown faces as “Unknown”.

- Process selected frames or use a smaller frame resolution to maintain real-time performance.

6. OpenCV Pipeline

Camera → Frame Acquisition → Resize → Grayscale/CLAHE → Face Detection → Face Alignment →

Feature Extraction → Feature Matching → Threshold Check → Identification → Display 7. Output

- Detected face bounding boxes.

- Student/staff name or ID.

- “Unknown” for unregistered persons.

- Real-time security display.

### 8. Justification

Grayscale conversion reduces computation, while CLAHE improves performance under uneven lighting. Face detection locates only relevant regions, reducing processing time. LBPH is suitable for low-cost systems because it is computationally lightweight. A confidence threshold prevents uncertain matches from being accepted. Frame resizing and processing selected frames improve real-time performance.

# 2. Intelligent Traffic Violation Detection System

## 1. Problem Statement

A traffic department wants to automatically detect, track, count, and analyze vehicles from roadside CCTV cameras and generate real-time alerts for vehicles crossing predefined lines or showing abnormal movement.

## 2. Objective

To develop an OpenCV-based traffic monitoring system that accurately detects and tracks vehicles while minimizing counting errors.

## 3. Constraints

- Multiple vehicles may appear simultaneously.

- Vehicles may overlap.

- Shadows can be detected as objects.

- Camera noise and illumination changes.

- Dynamic backgrounds.

- Vehicles must be tracked across frames.

- Real-time processing is required.

## 4. Input

- Live CCTV/video stream.

- Predefined virtual counting line.

- Region of interest.

- Vehicle detection parameters.

## 5. Processing / Algorithm

- Capture video frames using OpenCV.

- Resize frames for faster processing.

- Define the road region of interest.

- Convert frames into a suitable color space or grayscale. 5. Apply background subtraction such as MOG2 or KNN.

- Remove noise using Gaussian/median filtering.

- Apply morphological opening to remove small noise.

- Apply morphological closing to fill gaps in vehicle regions.

- Detect contours or use an object detector to identify vehicles.

- Calculate the centroid of each detected vehicle.

- Assign an ID to each vehicle.

- Track vehicles using centroid tracking, Kalman filtering, or another tracking method.

- Check whether the vehicle centroid crosses the predefined line.

- Increment the vehicle count only once for each vehicle.

- Analyze direction and unusual movements.

- Display vehicle IDs, count, movement direction, and violation alerts.

6. OpenCV Pipeline

## Figure: OpenCV-based system diagram

CCTV → Frame Acquisition → ROI Selection → Preprocessing → Background Subtraction/Object

Detection → Morphological Filtering → Contour Detection → Vehicle Tracking → Line Crossing

Detection → Violation Detection → Visualization

### 7. Output

- Real-time vehicle count.

- Vehicle tracking IDs.

- Direction of movement.

- Line-crossing/violation alerts.

- Bounding boxes around vehicles.

### 8. Handling Errors

- Shadows: Use shadow detection in MOG2 and morphological processing.

- Camera noise: Apply Gaussian or median filtering.

- Dynamic background: Use adaptive background subtraction.

- Multiple vehicles: Assign unique tracking IDs.

- Overlapping vehicles: Maintain tracking using position and motion information.

- Duplicate counting: Count only when a tracked vehicle crosses the line for the first time.

### 9. Justification

ROI selection reduces unnecessary processing. Background subtraction detects moving vehicles efficiently, while morphological operations remove small noise. Tracking maintains vehicle identity between frames.

A virtual line and unique vehicle ID ensure that the same vehicle is counted only once.

# 3. Automated Document Processing System

## 1. Problem Statement

A company receives thousands of scanned documents containing blur, noise, uneven illumination, different font sizes, and unwanted backgrounds. The system must automatically enhance these documents and prepare them for accurate OCR.

## 2. Objective

To design an OpenCV image-processing pipeline that improves document quality and extracts readable text efficiently using OCR.

## 3. Constraints

- Blurred documents.

- Uneven illumination.

- Background noise.

- Different font sizes.

- Skewed documents.

- Large number of documents.

- Batch processing requirement.

## 4. Input

- Scanned document images.

- Images in JPG, PNG, TIFF, or similar formats.

## 5. Processing / Algorithm

- Read the document using OpenCV.

- Resize the image if required.

- Convert the image to grayscale.

- Apply Gaussian or median filtering to reduce noise.

- Correct uneven illumination using adaptive techniques.

- Apply thresholding.

- Use adaptive thresholding or Otsu thresholding to separate text from background.

- Apply morphological opening to remove small unwanted objects.

- Apply morphological closing to connect broken characters.

- Detect document boundaries using contours.

- Apply perspective transformation to correct document geometry.

- Segment text regions or lines.

- Prepare the enhanced image for OCR.

- Pass the processed image to an OCR engine.

- Store extracted text in a file/database.

- Repeat the process for all documents in batch mode.

6. OpenCV Pipeline

## Figure: OpenCV-based system diagram

Input Document → Grayscale → Noise Filtering → Illumination Correction → Thresholding → Morphological Operations → Segmentation → Contour Detection → Perspective Correction → OCR →

Text Storage

### 7. Output

- Enhanced document image.

- Segmented text regions.

- Extracted machine-readable text.

- Processed document files.

### 8. Role of Each Stage

- Grayscale: Reduces a color image to one intensity channel and decreases computation.

- Filtering: Removes noise and improves character clarity.

- Thresholding: Separates foreground text from the background.

- Morphological operations: Remove small noise and repair broken text.

- Segmentation: Separates text lines/regions for better OCR.

- Geometric correction: Straightens skewed or perspective-distorted documents.

- OCR: Converts processed characters into editable text.

### 9. Justification

Preprocessing improves the contrast and readability of characters before OCR. Adaptive thresholding handles uneven illumination better than a single global threshold. Morphological operations remove unwanted noise and connect broken characters. Perspective correction produces a properly aligned document. Batch processing allows thousands of documents to be processed automatically.

# 4. Real-Time Multi-Person Tracking and Attendance System

## 1. Problem Statement

A college wants to automatically record student attendance using classroom CCTV. The system must detect multiple students, track them across frames, recognize registered students, and prevent duplicate attendance.

## 2. Objective

To design an OpenCV-based face recognition and tracking system that records each student's attendance once while operating continuously in real time.

## 3. Constraints

- Multiple students in one frame.

- Head movement and different face orientations.

- Temporary face disappearance.

- Duplicate attendance must be avoided.

- Real-time processing.

- Limited computational resources.

## 4. Input

- Classroom CCTV video.

- Registered student face database.

- Student ID and name information.

## 5. Processing / Algorithm

- Capture frames from the classroom CCTV.

- Resize frames for faster processing.

- Detect all visible faces.

- Convert detected face regions to grayscale.

- Normalize and align each face.

- Extract facial features using LBPH or face embeddings.

- Compare features with the registered student database.

- Assign an identity to each recognized student.

- Create a unique tracking ID for every detected person.

- Track each person across consecutive frames.

- If a face temporarily disappears, maintain its identity for a limited number of frames.

- Confirm recognition using multiple consecutive frames.

- Check the attendance database before marking attendance.

- Mark a student present only if the student is not already recorded.

- Display bounding boxes, names, and attendance status.

- Continue processing until the CCTV session ends.

6. OpenCV Pipeline

## Figure: OpenCV-based system diagram

CCTV → Frame Acquisition → Face Detection → Preprocessing → Feature Extraction → Face

Recognition → Multi-Person Tracking → Identity Association → Attendance Verification → Attendance

Database

### 7. Output

- Detected student faces.

- Student names/IDs.

- Tracking IDs.

- Attendance status.

- Final attendance list.

### 8. Preventing Duplicate Attendance

If recognized student ID is not in attendance list → Mark Present.

If student ID is already present → Do not mark again.

### 9. Handling Temporary Disappearance

The tracker maintains the student's tracking ID for a predefined number of frames when the face is temporarily blocked or not visible. When the face becomes visible again, the identity can be associated with the existing tracking ID.

### 10. Justification

Face recognition provides student identity, while tracking prevents repeated recognition of the same person in consecutive frames. Multiple-frame confirmation reduces false recognition. Maintaining an attendance set prevents duplicate entries. Frame resizing and selective face processing reduce computational delay.

# 5. Smart Home Surveillance and Intrusion Detection

## 1. Problem Statement

A home security system continuously monitors an entrance using a fixed camera. It must detect significant movement, track moving objects, identify suspicious activity, and generate an alert when an unauthorized person enters a predefined region.

## 2. Objective

To develop a lightweight OpenCV surveillance system that detects meaningful motion and minimizes false alarms caused by noise, shadows, illumination changes, and dynamic backgrounds.

## 3. Constraints

- Continuous video monitoring.

- Limited computational resources.

- Camera noise.

- Shadows.

- Changing illumination.

- Moving trees or other background objects.

- Small unwanted movements.

- Real-time alert generation.

## 4. Input

- Fixed CCTV/webcam video stream.

- Predefined restricted region.

- Motion detection parameters.

- Optional authorized-person database.

## 5. Processing / Algorithm

- Capture video frames using OpenCV.

- Resize the frames for efficient processing.

- Apply background subtraction using MOG2 or KNN.

- Generate a foreground mask.

- Apply Gaussian/median filtering to remove noise.

- Apply morphological opening to remove small objects.

- Apply morphological closing to fill gaps in detected objects.

- Find contours in the foreground mask.

- Remove contours below a minimum area threshold.

- Calculate bounding boxes and centroids of moving objects.

- Track detected objects across consecutive frames.

- Define a restricted region at the entrance.

- Check whether a tracked object enters the restricted region.

- Analyze movement duration and direction to distinguish meaningful activity.

- If an unauthorized person enters the region, generate an alert.

- Display the video, bounding boxes, tracking IDs, and alert status.

6. OpenCV Pipeline

## Figure: OpenCV-based system diagram

Camera → Frame Acquisition → Background Subtraction → Filtering → Morphological Operations → Contour Detection → Object Detection → Object Tracking → Restricted Region Check → Intrusion

Detection → Alert

### 7. Output

- Motion detection result.

- Moving-object bounding boxes.

- Tracking IDs.

- Intrusion status.

- Real-time security alert.

### 8. Reducing False Alarms

- Camera noise: Apply Gaussian/median filtering.

- Small background changes: Ignore contours below a minimum area.

- Shadows: Use shadow detection and color/intensity analysis.

- Illumination changes: Use adaptive background modeling.

- Trees/moving objects: Apply ROI restrictions and persistence checks.

- Temporary movement: Require movement to remain for several frames before generating an alert.

### 9. Justification

Background subtraction efficiently identifies moving regions in a fixed-camera environment. Filtering and morphological operations remove noise and improve object shapes. Contour area filtering prevents small movements from triggering alarms. Tracking provides information about object movement over time. A predefined restricted region ensures that alerts are generated only when meaningful movement occurs.